UCI SECOM Dataset

Semiconductor Manufacturing Process

Data Science Improving Manufacturing Quality

Randall Crawford, Graduate Student

Department of Data Science

National University

San Diego, CA  92123

r.crawford2194@student.nu.edu

Abstract

A complex modern semi-conductor manufacturing process is normally under consistent surveillance via the monitoring of signals/variables collected from sensors and or process measurement points. However, not all of these signals are equally valuable in a specific monitoring system. The measured signals contain a combination of useful information, irrelevant information as well as noise. It is often the case that useful information is buried in the latter two. Engineers typically have a much larger number of signals than are actually required. If we consider each type of signal as a feature, then feature selection may be applied to identify the most relevant signals. Process and Quality Engineers may then use these signals to determine key factors contributing to yield excursions downstream in the process. This will enable an increase in process throughput, decreased time to learning and reduce the per unit production costs.

To enhance current business improvement techniques the application of feature selection as an intelligent systems technique is being investigated.

1 Introduction

The evolution of manufacturing process control has been remarkable.  Advancement in control systems and detection tools are offering manufacturers valuable data to understand, monitor, maintain, control, improve, and optimize their processes.  The semiconductor manufacturing sector is a burgeoning example of the absolute need for these systems and tools.  Quality methodologies like Six Sigma can be more effectively implemented to this end.

The specific objective of this analysis is to help predict defect detection in a semiconductor manufacturing process by developing a robust machine learning model using the hypothetical SECOM dataset, leveraging strategic data preprocessing, various advanced feature selection techniques to identify the most predictive features, and the optimization of a XGBoost classifier to achieve a relatively high target F1-score, ensuring a balance of simplicity, interpretability, and high performance for practical deployment.

2 Dataset Review

The dataset was acquired from the UC Irvine Machine Learning Repository, and was from a semi-conductor manufacturing process.  Dimensionally, it has 1,567 records and 592 total features.  Each record represents a single production entity with associated measured features and a simple pass/fail yield for in house line testing.  In the Target column, “ –1” corresponds to a pass and “1” corresponds to a failure, and the Time stamp column is for that specific test point.  This leaves 590 features potentially derived from process data collection for each production entity.  The features are labeled numerically 1 through 590 and intentionally have no dictionary describing what they are specifically to protect the manufacturers’ interests.  It was created by Micheal McCann and Adrian Johnston, donated on 11/18/2008, and is licensed under a Creative Commons Attribution 4.0 International license.

3 Initial Considerations

In careful review of what little information provided concerning ‘Variable Information’:

As with any real-life data situation, this data contains null values (NaN) varying in intensity depending on the individual features. This needs to be taken into consideration when investigating the data either through pre-processing or within the technique applied.

It seemed appropriate to employ strategic data preprocessing to understand the nature of missingness in features.  Passes and failures could be related to feature sensor functionality.  Even though we do not understand what proper feature sensor function actually is, we can make some relational inferences from the existence of missingness and specific outliers (zero values) to our pass/fail outcome.  Feature removal, indicator and calculated variable creation, and imputation strategy can all be affected by these relational inferences.  Consequently, those activities can have an effect on feature reduction method results, and it turn, classification model parameters and results.

4 Strategic Data Preprocessing

The initial steps involved verifying data types, generating descriptive statistics, creating a Y dataset from the Target variable, creating a X dataset from the 590 feature variables, and redefining the Y values to binary (0 for pass, 1 for fail).  General statistics concerning missing values in X were calculated (Total features with missing values: 538, Total missing values: 41951, Total proportion of missing values: 0.045 or 4.5%).  When reviewing the descriptive statistics, it became apparent that X features have similar numbers of missing values.  By way of investigation, I decided to group these features by number of missing values.  Features with missing values played out into 22 distinct layers (Layer_1NA, Layer_2NA, Layer_3NA, Layer_4NA, Layer_5NA, Layer_6NA, Layer_7NA, Layer_8NA, Layer_9NA, Layer_10NA, Layer_12NA, Layer_14NA, Layer_24NA, Layer_51NA, Layer_260NA, Layer_273NA, Layer_715NA, Layer_794NA, Layer_949NA, Layer_1018NA, Layer_1341NA, Layer_1429NA), possibly indicating some other association between the features in each layer.

Another issue gathered from viewing the descriptive statistics was the existence of constant values in the dataset.  By evaluating each feature’s standard deviation for a zero value, I was able to generate a list of features for potential removal, given that constant value would likely have little to no predictive value.  In addition, the constant value list was verified by checking to see if the minimum and maximum values were the same.  Before the constant value features could be removed from X, each and its respective missingness needed to be evaluated for potential predictiveness on Y.

A chi-square test was performed for feature constant value distribution vs. expected baseline for pass/fail.   The expected baseline for pass/fail is based on the total number of failures (104) divided by total number of records (1,567) equaling 0.0664 or 6.64%.  A chi-square test was performed for constant feature missingness vs. Y.  The identified 116 constant value features and their missingness were not predictive (lowest p-value of 0.3665 > 0.05), and were removed from X.  ['6', '14', '43', '50', '53', '70', '98', '142', '150', '179', '180', '187', '190', '191', '192', '193', '194', '195', '227', '230', '231', '232', '233', '234', '235', '236', '237', '238', '241', '242', '243', '244', '257', '258', '259', '260', '261', '262', '263', '264', '265', '266', '267', '277', '285', '314', '315', '316', '323', '326', '327', '328', '329', '330', '331', '365', '370', '371', '372', '373', '374', '375', '376', '379', '380', '381', '382', '395', '396', '397', '398', '399', '400', '401', '402', '403', '404', '405', '415', '423', '450', '451', '452', '459', '462', '463', '464', '465', '466', '467', '482', '499', '502', '503', '504', '505', '506', '507', '508', '509', '510', '513', '514', '515', '516', '529', '530', '531', '532', '533', '534', '535', '536', '537', '538', '539']

This reduced X to 474 total features, 422 features with missing values, and 86.78 average missing values per feature.  Regrouped these features by missing values, since 116 had been removed.  The 22 distinct layers remained the same.  At this point, it seemed the appropriate time to check predictive missingness of all the remaining features in the reduced X by using a chi-square test on feature missingness vs. Y.  There were 52 features identified with predictive missingness.  ['22', '23', '24', '25', '26', '27', '28', '29', '30', '31', '32', '73', '74', '113', '160', '161', '162', '163', '164', '165', '166', '167', '168', '169', '170', '248', '295', '296', '297', '298', '299', '300', '301', '302', '303', '304', '305', '346', '347', '386', '431', '432', '433', '434', '435', '436', '437', '438', '439', '440', '441', '520']

Regenerated descriptive statistics for pass, failure, and combined for the reduced X dataset for ongoing reference.  Realizing that some of these missingness layers had an extremely high missing value count, it seemed useful to create a visualization to better understand their use chronologically while noting pass and fail status.

When evaluating this visualization, it appeared that all features were available, but some features of the high missingness layers (Layer_715NA, Layer_794NA, Layer_1018NA, Layer_1341NA) ramped up their utilization after sample 800.  It is hard to know why some of these features were more sparingly utilized before sample 800, but they were.  One key takeaway from this visualization was that missingness indicators should only be created for features with predictive missingness that are at Layer_260NA or above.  With that in mind, 8 feature missingness indicator variables were created for the features in Layer_715NA and Layer_794NA (missing_73, missing_74, missing_133, missing_248, missing_346, missing_347, missing_386, missing_520).  Furthermore, an indicator variable was created to represent the late utilization of the layers 949NA, 1018NA, and 1341NA, after sample record 800 (missing_late_utilization).  And lastly, a variable calculating the proportion of NaN features in a sample record was created (proportion_na_features).

Features associated with layers 1341NA and 1429NA were removed, given their insufficient number of real values to provide some meaningful predictive influence.  At this point, the reduced X dataset of 474 features had added 10 features (indicators and such) and removed 8 features (with extremely high NaN counts), equaling 476 total features.  Addressing the zero count in features, a feature with greater than or equal to 1341 zero values threshold was used, given their insufficient number of non-zero values to provide some meaningful predictive influence. 10 features were removed. [‘75’, ’115’, ‘207’, ‘210’, ‘250’, ‘343’, ‘348’, ‘388’, ‘479’, ‘522’] This brings the revised feature count for the further reduced X dataset to 466.

While reviewing descriptive statistics further, it was noticed that some minimum and maximum zero values did not make any sense with their feature distribution.  The decision was made not to touch outliers, because the nature of features was not understood.  Although, an effort was made to address some of the zeros by converting them to NaN values based on at least one of three criteria, prior to imputation.

1st: Feature distributions with a minimum zero value and significant left skew (<-0.5)

2nd: Feature distributions with a negative minimum value, a maximum zero value, and significant right skew (>0.5)

3rd: If zero outside feature distribution (IQR*1.5), with the exception of less than an absolute maximum 0.05 value.

*10 features were converted. [‘4’, ‘8’, ‘22’, ‘23’, ‘26’, ‘27’, ‘28’, ‘113’, ‘129’, ‘417’]

Different imputation methods were either considered or explored (Random Forest, Linear Regression, KNN Imputation, Median, Mean, MICE Imputation), but given the dataset had linear and non-linear features with significantly varying distributions, a feature pass and failure mean strategy directly related to the outcome variable made the most sense.  Imputed all features in layers from 1NA to 794NA with feature pass and failure means.  For features in Layer_949NA and Layer_1018NA, a hybrid approach was used.  For sample records < 800, they were imputed with the feature mean to purposely differentiate them from sample records >= 800 that were imputed with feature pass and failure means.  Verified that all remaining features had been imputed and no NaN values were present final X dataset.

5 Feature Selection

Initially, use seven different feature selection methods to reduce the final X dataset (476) to 100 features (SelectKBest, Mutual Information, Recursive Feature Elimination (RFE) with Logistic Regression, Recursive Feature Elimination (RFE) with XGBoost Classifier, Random Forest Method, L1 Regularization (Lasso), Variance Threshold + SelectKBest).

  - SelectKBest Method

    - Selected 100 features: ['15', '22', '23', '27', '29', '33', '34', '39', '41', '57', '59', '60', '64', '65', '66', '69', '71', '77', '80', '91', '96', '101', '104', '112', '113', '122', '123', '124', '125', '126', '127', '128', '130', '131', '134', '160', '161', '164', '165', '166', '167', '181', '184', '197', '198', '200', '201', '206', '211', '248', '281', '295', '296', '299', '300', '301', '317', '320', '338', '349', '366', '431', '432', '435', '436', '437', '438', '453', '456', '461', '469', '470', '472', '478', '511', '512', '520', '543', '544', '548', '551', '552', '554', '555', '557', '558', '563', '566', '568', '570', '574', '576', 'missing_73', 'missing_74', 'missing_113', 'missing_248', 'missing_346', 'missing_347', 'missing_386', 'missing_520’]

  - Mutual Information Method

    - Selected 100 features: ['3', '7', '20', '34', '35', '39', '41', '42', '56', '58', '66', '71', '73', '74', '89', '110', '111', '113', '120', '122', '123', '126', '127', '128', '129', '131', '135', '139', '226', '245', '246', '247', '248', '269', '274', '275', '276', '278', '289', '308', '309', '311', '332', '346', '347', '383', '385', '386', '406', '407', '408', '412', '413', '414', '417', '429', '444', '447', '448', '468', '478', '511', '518', '519', '520', '540', '542', '547', '548', '549', '550', '551', '552', '553', '554', '555', '556', '557', '558', '563', '564', '565', '566', '567', '568', '569', '570', '571', '572', '574', '575', '576', '577', '578', '579', '580', '581', '582', '584', '590’]

  - Recursive Feature Elimination (RFE) with Logistic Regression

    - Selected 100 features: ['15', '20', '21', '27', '32', '33', '35', '36', '37', '38', '48', '59', '60', '65', '66', '76', '78', '82', '84', '87', '97', '99', '100', '103', '109', '112', '118', '122', '125', '127', '129', '130', '131', '133', '141', '143', '146', '147', '151', '157', '173', '175', '176', '178', '185', '196', '203', '205', '212', '214', '215', '217', '218', '219', '248', '269', '270', '278', '286', '291', '306', '311', '313', '317', '319', '322', '334', '337', '339', '341', '349', '350', '352', '356', '391', '407', '412', '417', '425', '438', '444', '446', '447', '449', '455', '456', '458', '476', '517', '525', '546', '547', '553', '557', '558', '564', '565', '567', '572', 'missing_113']

  - Recursive Feature Elimination (RFE) with XGBoost Classifier

    - Selected 100 features: ['1', '11', '17', '22', '30', '34', '49', '60', '65', '68', '73', '74', '79', '80', '82', '85', '89', '92', '93', '96', '97', '103', '104', '113', '116', '118', '119', '121', '125', '126', '128', '130', '134', '139', '141', '147', '151', '156', '164', '173', '185', '189', '197', '215', '219', '239', '240', '248', '249', '253', '280', '284', '292', '296', '299', '321', '346', '347', '362', '363', '368', '386', '394', '417', '424', '427', '429', '431', '434', '438', '469', '470', '478', '489', '512', '520', '524', '547', '548', '549', '550', '552', '555', '556', '558', '562', '563', '564', '565', '567', '568', '569', '570', '576', '580', '581', '582', '588', 'missing_73', 'missing_113’]

  - Random Forest Method

    - Selected 100 features: ['248', '520', '386', '113', '347', 'missing_520', '74', 'missing_73', 'missing_347', 'missing_74', '346', '563', '581', '60', '565', '73', '478', '65', 'missing_346', '580', '427', '349', '566', '567', 'missing_113', '41', '133', '552', '342', '66', '570', '206', '555', '268', '63', '153', '558', '589', 'missing_386', '568', '438', '56', '249', '442', '289', '569', '3', '511', '52', '299', '68', '64', '332', '188', '540', '27', '412', '426', '406', 'missing_248', '154', '29', '164', '288', '5', '383', '121', '461', '208', '474', '28', '204', '108', '81', '495', '302', '353', '148', '578', '10', '445', '79', '518', '125', '338', '22', '71', '421', '524', '37', '200', '411', '120', '362', '39', '166', '122', '139', '112', '430’]

  - L1 Regularization (Lasso) Method

    - Selected 100 features: ['1', '2', '3', '4', '15', '16', '22', '23', '24', '25', '41', '47', '52', '56', '60', '63', '65', '68', '69', '71', '72', '73', '74', '89', '91', '116', '134', '136', '137', '138', '139', '141', '152', '153', '160', '162', '163', '184', '188', '189', '201', '205', '209', '224', '226', '251', '253', '269', '275', '295', '296', '297', '298', '341', '364', '414', '419', '420', '424', '426', '429', '432', '433', '434', '437', '439', '440', '457', '461', '468', '469', '472', '473', '474', '478', '483', '484', '485', '486', '487', '488', '489', '490', '491', '495', '497', '500', '501', '511', '512', '520', '542', '548', '556', '562', '563', '570', '573', '578', '582']

  - Variance Threshold + SelectKBest

    - Selected 100 features: ['15', '22', '23', '27', '29', '32', '33', '34', '39', '41', '60', '64', '65', '66', '69', '71', '91', '112', '116', '123', '124', '125', '126', '127', '128', '130', '134', '139', '160', '161', '164', '165', '166', '167', '176', '181', '182', '184', '189', '197', '198', '200', '201', '204', '206', '295', '296', '300', '301', '302', '317', '320', '334', '338', '342', '412', '417', '418', '424', '431', '432', '434', '435', '436', '437', '438', '453', '456', '461', '469', '470', '472', '476', '478', '485', '489', '511', '512', '520', '546', '548', '551', '552', '554', '555', '557', '558', '563', '570', '574', '578', 'missing_73', 'missing_74', 'missing_113', 'missing_248', 'missing_346', 'missing_347', 'missing_386', 'missing_520', 'missing_late_utilization']

At this point in the process, the idea of using Synthetic Minority Sampling Technique (SMOTE) was considered, but ruled out due to the desire to preserve the impact of real-life dataset provided.  Out of curiosity for each method feature set, Variance Inflation Factor (VIF) analysis was performed followed by Backward Elimination with Logistic Regression to determine best LR model and corresponding F1-Score.

- SelectKBest

  - Variance Inflation Factor (VIF) Results: removed 57 features due to high VIF (>10), ['27', '71', '122', '123', '124', '128', '160', '161', '164', '165', '166', '167', '181', '184', '197', '198', '200', '206', '211', '248', '295', '296', '299', '300', '301', '320', '349', '431', '432', '435', '436', '437', '438', '453', '456', '470', '472', '478', '520', '551', '552', '554', '555', '557', '558', '566', '568', '574', '576', 'missing_73', 'missing_74', 'missing_113', 'missing_248', 'missing_346', 'missing_347', 'missing_386', 'missing_520']

  - Backward Elimination with Logistic Regression Results: 13 significant features, ['15', '22', '57', '60', '64', '65', '101', '112', '125', '130', '281', '512', '570'], Pseudo R-square: 0.1678, Log-Likelihood:       -318.36

- F1-Score with significant features: 0.2197

•	Recursive Feature Elimination (RFE) with Logistic Regression

- Variance Inflation Factor (VIF) Results: removed 42 features due to high VIF (>10), ['35', '37', '38', '66', '118', '143', '151', '173', '175', '176', '178', '203', '205', '212', '214', '218', '269', '278', '286', '311', '313', '319', '322', '334', '337', '339', '341', '350', '352', '356', '391', '407', '444', '446', '447', '449', '455', '458', '476', '525', '565', '567’]

- Backward Elimination with Logistic Regression Results: 18 significant features, ['15', '27', '32', '33', '36', '59', '60', '65', '76', '100', '112', '122', '130', '147', '412', '417', '438', '558’], Pseudo R-square: 0.1954, Log-Likelihood: -307.81

- F1-Score with significant features: 0.2576

•	Recursive Feature Elimination (RFE) with XGBoost

- Variance Inflation Factor (VIF) Results: removed 27 features due to high VIF (>10), ['17', '118', '156', '164', '185', '248', '253', '284', '299', '321', '346', '347', '386', '429', '478', '520', '550', '552', '555', '558', '565', '567', '568', '569', '570', '580', '581']

- Backward Elimination with Logistic Regression Results: 14 significant features, ['22', '60', '65', '74', '79', '125', '130', '147', '417', '431', '438', '512', '563', '576’], Pseudo R-square: 0.1916, Log-Likelihood: -309.26

- F1-Score with significant features: 0.2604

•	L1 Regularization (Lasso)

- Variance Inflation Factor (VIF) Results: removed 21 features due to high VIF (>10), ['47', '68', '71', '141', '153', '160', '162', '163', '205', '226', '295', '296', '297', '298', '341', '364', '414', '426', '432', '437', '478']

- Backward Elimination with Logistic Regression Results: 9 significant features, ['15', '22', '60', '65', '74', '134', '434', '512', '542’], Pseudo R-square: 0.1377, Log-Likelihood: -329.88

- F1-Score with significant features: 0.2233

•	Variance + SelectKBest

- Variance Inflation Factor (VIF) Results: removed 59 features due to high VIF (>10), ['22', '27', '64', '65', '66', '71', '123', '128', '139', '160', '161', '164', '165', '166', '167', '181', '184', '197', '198', '200', '204', '206', '295', '296', '300', '301', '302', '320', '334', '342', '412', '431', '432', '435', '436', '437', '438', '453', '456', '470', '472', '476', '478', '551', '552', '554', '555', '557', '558', '574', '578', 'missing_73', 'missing_74', 'missing_113', 'missing_248', 'missing_346', 'missing_347', 'missing_386', 'missing_520']

- Backward Elimination with Logistic Regression Results: 14 significant features, ['15', '23', '60', '112', '125', '130', '176', '317', '417', '461', '489', '511', '512', '570’], Pseudo R-square: 0.1551, Log-Likelihood: -323.22

- F1-Score with significant features: 0.2729

Used the Random Forest Classifier with the Mutual Information method feature set to determine the top 20 most important features and corresponding F1-Score, preserving potential predictive non-linear features.

Feature  Importance

520         0.065658

248         0.052710

386         0.044980

346         0.038994

347         0.031049

113         0.027171

73           0.024113

74           0.019468

66           0.018837

555         0.018163

566         0.018128

563         0.017516

540         0.015816

580         0.014260

478         0.013752

552         0.013747

581         0.013727

570         0.013474

565         0.013002

558         0.012526

F1-score with Random Forest on Mutual Info features: 0.2646

6 Machine Learning Model Creation with XGBoost Classifier

Trained models utilizing XGBoost Classifier for three of the previously used feature sets (Mutual Info, RFE, and SelectKBest) and introduce three alternative feature selection methods (Boruta, SHAP, and L1 via XGBoost).

- Boruta Selection Method

  - Selected 28 features: ['17', '22', '34', '60', '65', '68', '74', '96', '113', '125', '141', '151', '240', '248', '284', '346', '347', '424', '427', '524', '555', '563', '565', '569', '570', '581', 'missing_73', 'missing_113']

- SHAP Selection Method

  - Selected 100 features: ['113', '346', '248', '60', '74', '73', '581', '569', '563', 'missing_73', '556', '564', '11', '582', '558', '386', '1', '347', '548', '570', '22', 'missing_113', '470', '567', '164', '478', '299', '91', '520', '512', '308', '461', '572', '160', '547', '249', '576', '492', '103', '313', '568', '555', '134', '554', '46', '301', '163', '487', '566', '206', '182', '432', '549', '352', '173', '580', '426', '185', '288', '128', '552', '15', '87', '272', '284', '114', '17', '557', '172', '297', '485', '104', '96', '292', '41', '117', '427', '80', '565', '447', '141', '153', '551', '588', '589', '489', '161', '123', '147', '438', '151', '125', '34', '146', '19', '333', '5', '181', '420', '240']

- L1 via XGBoost Selection Method

  - Selected 100 features: ['113', '346', '248', '60', '74', '73', '581', '569', '563', 'missing_73', '556', '564', '11', '582', '558', '386', '1', '347', '548', '570', '22', 'missing_113', '470', '567', '164', '478', '299', '91', '520', '512', '308', '461', '572', '160', '547', '249', '576', '492', '103', '313', '568', '555', '134', '554', '46', '301', '163', '487', '566', '206', '182', '432', '549', '352', '173', '580', '426', '185', '288', '128', '552', '15', '87', '272', '284', '114', '17', '557', '172', '297', '485', '104', '96', '292', '41', '117', '427', '80', '565', '447', '141', '153', '551', '588', '589', '489', '161', '123', '147', '438', '151', '125', '34', '146', '19', '333', '5', '181', '420', '240']

The modeling results are noted in the following table:

|  | Mutual Info | RFE | SelectKBest | Boruta | SHAP | L1 via XGBoost |
| --- | --- | --- | --- | --- | --- | --- |
| # of Features | 100 | 100 | 100 | 28 | 100 | 100 |
| Best Threshold | 0.15 | 0.40 | 0.40 | 0.15 | 0.25 | 0.15 |
| Precision | 0.818 | 0.877 | 0.934 | 0.792 | 0.863 | 0.822 |
| Recall | 0.692 | 0.481 | 0.548 | 0.731 | 0.663 | 0.712 |
| F1-Score | 0.750 | 0.621 | 0.691 | 0.760 | 0.750 | 0.763 |

Initial results were promising for all feature reduced datasets, due to the suitability of the XGBoost Classifier as a modeling tool.  Most of the thresholds were on the lower side, but the precision results were greater than the recall results.  After establishing some baseline results for the feature sets, it was time to endeavor to optimize them with GridSearchCV to determine appropriate grid parameters for the XGBoost Classifier.  Each feature set was carefully iterated with 5-StratifiedKFold cross validation, and the eight optimal parameter results are noted in the table below:

|  | Mutual Info | RFE | SelectKBest | Boruta | SHAP | L1 via XGBoost |
| --- | --- | --- | --- | --- | --- | --- |
| ‘max_depth’ | 6 | 3 | 3 | 3 | 3 | 3 |
| ‘learning_rate’ | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 | 0.1 |
| ‘n_estimators’ | 120 | 120 | 50 | 200 | 260 | 250 |
| ‘scale_pos_weight’ | 14 | 14 | 14 | 14 | 14 | 14 |
| ‘reg_alpha’ | 0 | 0.1 | 0 | 0 | 0.1 | 0 |
| ‘reg_lambda’ | 0.1 | 0.1 | 0.1 | 0 | 0.1 | 0 |
| ‘gamma’ | 0 | 0 | 0 | 0 | 0 | 0.3 |
| ‘subsample’ | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

The optimized modeling results are noted in the following table:

|  | Mutual Info | RFE | SelectKBest | Boruta | SHAP | L1 via XGBoost |
| --- | --- | --- | --- | --- | --- | --- |
| # of Features | 100 | 100 | 100 | 28 | 100 | 100 |
| F1-Score | 0.731 | 0.680 | 0.738 | 0.804 | 0.792 | 0.789 |
| Best Threshold | 0.35 | 0.55 | 0.50 | 0.55 | 0.50 | 0.50 |
| Precision | 0.831 | 0.847 | 0.831 | 0.926 | 0.874 | 0.902 |
| Recall | 0.712 | 0.587 | 0.663 | 0.721 | 0.731 | 0.712 |
| F1-Score (Tuned) | 0.767 | 0.693 | 0.738 | 0.811 | 0.796 | 0.796 |

Optimized results showed an improvement with thresholds and F1-scores across the board.  The Boruta feature set seemed to garner the best results.  The relevant contingency table and threshold tuning metrics for Boruta results are below:

| At 0.55 Threshold | Predicted Pass | Predicted Failure |
| --- | --- | --- |
| Actual Pass (Y=0) | 1457 | 6 |
| Actual Failure (Y=1) | 29 | 75 |

Special Note: A train-test split (80/20) was not used in this analysis.  It maximized the data available for training in the 5-fold cross validation, which is critical for a small, imbalanced dataset like SECOM, leading to a more stable evaluation.  Cross-validation achieves the goal of assessing generalization without sacrificing data, making a separate test less necessary in this case. This avoids potential misalignment between feature selection and evaluation, ensuring the reported F1-Score reflects the true performance of the selected features.

Further exploration was done concerning combining datasets, after generating overlay information concerning feature sharing between methods.  Overlay information below:

Core Features (selected by all methods): 2 features ['125', '248']

Features selected by at least two methods: 131 features

Features unique to Boruta: 0 features

Features unique to SHAP: 22 features

Features unique to L1: 23 features

Features unique to MI: 34 features

Features unique to SelectKBest: 35 features

Features unique to RFE: 38 features

Overlap between Boruta and SHAP: 24 features

Overlap between Boruta and L1: 25 features

Overlap between Boruta and MI: 13 features

Overlap between Boruta and SelectKBest: 13 features

Overlap between Boruta and RFE: 8 features

Overlap between SHAP and SelectKBest: 43 features

Overlap between L1 and SHAP: 47 features

Overlap between L1 and MI: 34 features

Overlap between L1 and SelectKBest: 36 features

Overlap between L1 and RFE: 32 features

Overlap between MI and SHAP: 38 features

Overlap between MI and SelectKBest: 31 features

Overlap between MI and RFE: 30 features

Overlap between RFE and SHAP: 24 features

Overlap between RFE and SelectKBest: 21 features

F1-Scores generated from Boruta and strategic feature combinations below:

Boruta (28 features): 0.811

Features Selected by At Least Two Methods (131 features): 0.802

Boruta + SHAP + L1 (153 features): 0.790

Boruta + Unique SHAP + Unique L1 (127 features): 0.768

Boruta + SHAP ∩ L1 (~54 features): 0.802

Boruta + Features Selected by At Least 3 Methods (70 features): 0.776

Boruta + SHAP ∩ L1 + At Least 4 Methods (57 features): 0.776

Boruta + Top 11 from SHAP ∩ L1 (39 features): 0.800

7 Summation

The first part of the analysis was a serious effort in strategic data preprocessing of an older and well-studied dataset full of constant values, missingness, zero values, and distribution variation. The anonymous nature of the features led one to try to pull inferences from those complexities.  Constant values probably indicated that a specific process was operational during a product run.  Missingness could be related to not using a specific process that may not have been necessary for a product type, or could be related to special processes to be only used under specific circumstances.  Zero values could easily be justified depending on what sensor information is being monitored, but could also be an indicator that a sensor is not reading properly.  The distribution variation indicates that data is being collected from all types of processes.

The second part of the analysis related to the application of various feature selection methods.  The third part of the analysis related to use of XGBoost Classifier to identify an appropriate feature method for this SECOM dataset, and Boruta proved itself formidable.